##### SCOPE

This memo is an amendment to ME-T-NT-110056 Is. A “Program FITAN, version 3. User's manual.”. It is intended to provide a temporary solution (before the software tool reporting is improved) to extract the stresses in base and walls of tension fittings, used in intermediate calculations within FITAN.

##### REFERENCES

- Document Reference	Issue	Title

NT-00-ECE-063	Is. A	Procedimiento de Cálculo de Angulares y Herrajes

[E. F. Bruhn]	1973	ANALYSIS AND DESIGN OF FLIGHT VEHICLE STRUCTURES

##### BENDING STRESS IN BASE OF TENSION FITTING

The value of the “fictitious” applied stress Fbue (*) in the base (plate where the tension bolt is installed), according to the FITAN method, can be calculated as follows…

Reserve Factor for base bending RFb from “fictitious” allowable and applied stresses Fbu and Fbue the additional safety factor jmu (typ. the fitting factor), using FITAN and Ref. 1 notation are:

RFb = Fbu / (jmu · Fbue)	→	Fbue = Fbu / (jmu · RFb)

Cozzone ratio definition (per Bruhn, C3.4, shape factor k =1.5):

f0/fm = f0 / Ftu = 2 · (Fbu / Ftu - 1) 	→	Fbu = Ftu · (1 + f0/fm / 2)

Thus, the applied “fictitious” stress Fbue at base can be computed from input variables as:

Fbue = Ftu · (1 + f0/fm / 2) / (jmu · RFb)

(*) Fbu and Fbue are the "fictitious" stresses calculated using linear/elastic beams bending theory even beyond the yield stress (Cozzone’s method, see Bruhn Ref. 2, chapter C3.4,). They can exceed Ftu once material reaches the inelastic/plastic regime. Under fatigue loads, typically below yield, they are equivalent to the classical bending stresses.

Alternatively, not being official methods, for T and L tension fittings some spreadsheets are available with more verbose reporting and the explanations of the method details included (see attachments).

##### STRESS IN WALLS OF TENSION FITTING

At certain distance from base, the wall can be treated as a beam section. For symmetrical T and for L fittings, stress at wall is the sum of axial forces and bending (due to eccentricity). Thus…

- T-fitting (symmetrical)  →  σ = ΣP / (b·tw)    …where: P tension loads, b width, tw wall-thickness

- L-fitting (thick-wall) → σ = P / (b·tw) + Fbue·tb2/tw2  … Fbue base bending stress, tb base-thickness

… as σbending = 6·M/tw2 = 6·(Fbue·tb2/6) / tw2   .... M bending moment at base/wall crossing

For corner and channel fittings, the recommended solution to evaluate stress in that section of the walls is to run FLEXA software (see example annexes). This approach shall be representative of the real stress as far as fitting design does not have large differences in thickness between base and walls, nor large differences between the distances of bolt to each wall.

##### ANNEXES / ATTACHMENTS

- Spreadsheets for calculation of T and L fittings (not-official methods)

- Example of calculation of stress in corner fitting walls with FLEXA