##### ARES 5

For metal parts, ARES reports axial stress (σx) and shear stress (τxy) for each element and section.

The static margin is obtained as Reserve Factor comparing the material ultimate stress (Ftu) with the Von Mises stress (σvm), where this is defined, for the quasi-unidimensional ARES problem (σy=0), as:

σvm = ( σx2 + 3·τxy2 )0.5

If analysis requires the principal stresses, the following expressions can be used to do the transformation:

σ1 = σx/2 + [ (σx/2)2  + τxy2 ]0.5

σ2 = σx/2 – [ (σx/2)2  + τxy2 ]0.5

##### ARES 4

In the previous version, ARES reported the Von Mises stress (one data column instead of 2 for stresses), but also the resultant section load (P), combining axial load (N) and shear load (S). Then, taking into account section width (w) and thickness (t), as…

σvm2 = σx2 + 3·τxy2

P2 = N2 + S2	-> [ P/(w·t) ]2 = [N/(w·t)]2 + [S(w·t)]2 = σx2 + τxy2

A simple linear system of two equations can be solved to get the axial and shear stresses:

σvm2 		= σx2 + 3·τxy2

[ P/(w·t) ]2 	= σx2 + τxy2

…

σx2 = { 3·[ P/(w·t) ]2 - σvm2 } / 2

τxy2 = { σvm2  - [ P/(w·t) ]2 } / 2

The transformation to principal stresses can be done as shown in previous section.